# HARDWARE DESIGN REVIEW

LAN7800  |  USB 3.1 Gen 1 to Gigabit Ethernet Adapter  |  Rev A

| DOCUMENT | REVISION | RELEASE CLASSIFICATION | PROJECT |
| --- | --- | --- | --- |
| LAN7800-HDR-001 | 1.1 | Production Approved — design-owner release | LAN7800 Rev A |

# 1. Design review summary

Rev A integrates a Microchip LAN7800 USB-to-Gigabit Ethernet controller, SuperSpeed USB Type-B interface, magnetics-equipped RJ45 connection, buck-regulated power distribution, high-speed ESD protection and a 25 MHz reference clock. The four-layer Altium PCB applies interface-specific routing rules and a controlled-impedance layer stack.

The issued manufacturing data comprises editable schematic and PCB source, a fabrication drawing, Gerber X2 and NC Drill outputs, a controlled BOM and component placement coordinates. The final Altium design-rule report records zero warnings and zero violations.

# 2. Design deliverables and configuration

| WORK PRODUCT | CONFIGURATION / LOCATION | REVIEW RECORD |
| --- | --- | --- |
| Altium source | Hardware/Altium/ — PrjPcb, two SchDoc sheets, PcbDoc, PcbLib | Configuration identified |
| Schematic release | Docs/LAN7800_USB_Ethernet_RevA_Schematic.pdf | Two functional sheets |
| Fabrication drawing | Manufacturing/LAN7800_USB_Ethernet_RevA_Fabrication.PDF | Outline, stack-up and drill table |
| Fabrication outputs | Manufacturing/GerberX2/ and Manufacturing/NC Drill/ | Four copper layers, masks, paste, drill |
| BOM / component placement | Manufacturing/BoM/ and Manufacturing/CPL/ | 22 BOM groups; 48 positions |

# 3. Requirements and design checks

| DISCIPLINE | DESIGN REVIEW RECORD | DISPOSITION |
| --- | --- | --- |
| USB interface | SuperSpeed and USB 2.0 signal paths; 90 Ω differential routing target. | Documented |
| Ethernet MDI | RJ45 magnetics; 100 Ω differential routing target and pair-length rules. | Documented |
| Power delivery | TLV62568 buck supply, power distribution, decoupling and protection. | Documented |
| Clock and resets | 25 MHz reference crystal and LAN7800 control connections. | Documented |
| Signal integrity | Dedicated differential-pair classes, impedance and matched-length constraints. | DRC checked |
| Fabrication | Four-layer construction, board profile, plated/non-plated drills, solder mask and paste. | Release data available |

# 4. Detailed technical review

| AREA | DESIGN DECISION / REVIEW NOTE |
| --- | --- |
| High-speed USB | Maintain the specified 90 Ω differential geometry, controlled pair spacing, closely coupled P/N routes and uninterrupted adjacent reference plane. Verify design rule profiles correspond to the issued stack-up. |
| Gigabit Ethernet | Maintain 100 Ω differential MDI routing; review magnetics centre-tap network, polarity and pin assignment against the LAN7800 and RJ45 specifications. |
| Power and thermal | Review TLV62568 power-stage loop geometry, inductor current margin, decoupling locations, return currents and output ripple allocation at the controller supply pins. |
| ESD / EMC | ESD7008 protection at USB interface; ensure discharge loops and connector-to-ground paths remain short and free of unnecessary stubs. |
| Oscillator | Confirm 25 MHz crystal parameters, load-capacitance assumptions, trace length, local ground and noise separation around oscillator pins. |
| DFM / assembly | Check solder-mask apertures, paste requirements, exposed pads, polarity marks, connector fit, pick-and-place origin, rotations and reflow compatibility. |
| DFT / inspection | Provide accessible rails and reset test points; define visual/continuity checks and functional USB/Ethernet commissioning sequence for first article. |

# 5. PCB design-rule verification

The Rev A fabrication drawing specifies a 70.00 × 34.00 mm PCB, nominal four-layer IS400/FR-4 construction, USB differential target 90 Ω ±10%, and Gigabit Ethernet differential target 100 Ω ±10%. Nominal stack legend and finished-thickness annotation differ by 0.02 mm (1.53 / 1.55 mm), within typical manufacturing stack tolerances pending the fabricator’s controlled stack specification.

The published Altium DRC report, dated 11 August 2026, records 0 warnings and 0 rule violations, including clearance, un-routed nets, hole rules and matched-length constraints. DRC records refer to the configured active rules and must be preserved with the issued revision.

# 6. Configuration and engineering follow-up

| ID | ENGINEERING REVIEW NOTE | RECOMMENDED DOCUMENT CONTROL ACTION |
| --- | --- | --- |
| ER-01 | C5 is identified as “C0603” in the existing schematic, with no electrical capacitance stated. | Record the selected capacitance on schematic and controlled BOM to maintain agreement. |
| ER-02 | Some component order codes were chosen during BOM preparation. | Retain approved sourcing records with voltage, tolerance, temperature and package requirements. |
| ER-03 | 22 µF MLCC groups include compact package selections. | Record effective-capacitance assessment under operating DC bias. |
| ER-04 | CPL includes 48 unique top-layer placements. | Maintain assembly origin/rotation convention and first-article placement checks. |

# 7. Approval and release record

Design-owner release classification: PRODUCTION APPROVED. This report identifies the controlled design outputs and the documented engineering review points for Rev A. Manufacturing planning and configuration management shall use the revision-matched schematic, PCB, BOM, CPL and fabrication outputs.

| RELEASE ROLE | RECORD | DATE |
| --- | --- | --- |
| Document prepared | CD Designs — Engineering | 10 Oct 2026 |
| Design-owner approval | Approval classification specified by design owner | 10 Oct 2026 |

Repository: https://github.com/CDonohoe-Designs/USB3-Gigabit-Ethernet-Adapter-Portfolio