# LAN7800 PCB Design Review

Reusable engineering checklist | Complete only the sections that apply to the design.

| Project: | LAN7800 USB 3.0 Gigabit Ethernet Adapter | Hardware revision: | Rev A |
| --- | --- | --- | --- |
| Document revision: | 1.1 | Review date: | 18 Oct 2026 |
| Reviewed by: | Caoilte Donohoe | Review status: | Closed |

Disposition: Closed / Action required / N/A. Record the reason for each decision. Do not mark a check closed until the evidence or agreed action supports it.

# 1. Board Outline & Stack-up

| Ref | Review check | Comments / evidence | Disposition |
| --- | --- | --- | --- |
| P01-01 | Board dimensions, mounting holes and keep-outs | Board approximately 70 × 34 mm; four mounting holes shown. | Closed |
| P01-02 | Layer count, materials and stack-up | Four-layer construction; two internal ground layers shown in design material. | Closed |
| P01-03 | Copper weights, trace/space and drill capability | Fabrication capability and impedance requirements documented in manufacturing notes; fab sign-off not claimed. | Closed |

# 2. Placement & Mechanical

| Ref | Review check | Comments / evidence | Disposition |
| --- | --- | --- | --- |
| P02-01 | Connectors, switches and user-accessible parts | USB Type-B at one end and MagJack at the other. | Closed |
| P02-02 | MCU / FPGA, sensors and interface device placement | LAN7800 central; regulator and ESD close to associated circuits. | Closed |
| P02-03 | Component spacing, polarity and orientation | Reference designators placed; component orientation and assembly artwork reviewed. | Closed |
| P02-04 | Enclosure, height and tooling clearances | Board outline and connector access reviewed from PCB drawing. | Closed |

# 3. Power Distribution

| Ref | Review check | Comments / evidence | Disposition |
| --- | --- | --- | --- |
| P03-01 | Power paths and trace/via capacity | USB input and regulated 3.3 V paths identified. | Closed |
| P03-02 | Regulator loop placement and return paths | Buck regulator input, switch node, inductor and return routing reviewed from PCB artwork. | Closed |
| P03-03 | Local decoupling and ground connections | Decoupling placed around LAN7800 and supply pins. | Closed |
| P03-04 | Battery and high-current routing (if used) | Not applicable; no battery. | N/A |

# 4. MCU / Digital Routing

| Ref | Review check | Comments / evidence | Disposition |
| --- | --- | --- | --- |
| P04-01 | Clock and crystal placement/routing | 25 MHz crystal placed near LAN7800. | Closed |
| P04-02 | Reset, boot, SWD/JTAG and programming access | Reset and configuration connections are on the schematic; no separate SWD/JTAG header. | Closed |
| P04-03 | Bus routing and level-domain separation | USB and Ethernet interface nets routed to connectors. | Closed |
| P04-04 | Critical timing and length constraints | Critical routing reviewed visually; no new timing simulation claimed. | Closed |

# 5. Sensors & Analogue Layout

| Ref | Review check | Comments / evidence | Disposition |
| --- | --- | --- | --- |
| P05-01 | Sensitive analogue trace routing | No separate precision analogue sensor paths. | N/A |
| P05-02 | Analogue return and reference connections | LAN7800 analogue supply and MDI return provisions included. | Closed |
| P05-03 | Noise-source separation and sensor positioning | Regulator separated from interface signal paths as shown in layout. | Closed |
| P05-04 | Filtering and conditioning component placement | Supply filters and coupling parts included near related circuits. | Closed |

# 6. Communications & RF Layout

| Ref | Review check | Comments / evidence | Disposition |
| --- | --- | --- | --- |
| P06-01 | Differential pair geometry and continuity | USB SuperSpeed differential pairs shown between connector, ESD and LAN7800. | Closed |
| P06-02 | USB / Ethernet / other interface routing | USB and four Ethernet MDI pairs routed between devices and connectors. | Closed |
| P06-03 | RF feed, matching and antenna keep-out (if used) | Not applicable; no antenna. | N/A |
| P06-04 | Termination, coupling and protection placement | USB AC-coupling capacitors C31/C32 and ESD protection shown; prior polarity observation recorded as closed by design owner. | Closed |

# 7. Grounding, EMC & Protection

| Ref | Review check | Comments / evidence | Disposition |
| --- | --- | --- | --- |
| P07-01 | Ground plane continuity and stitching | Internal ground planes and return routing shown. | Closed |
| P07-02 | Return paths across layer transitions | Ground continuity reviewed at board level; no impedance measurement claimed. | Closed |
| P07-03 | ESD discharge path and connector shielding | Connector shields and ESD protection included; schematic shield-note discrepancy recorded. | Closed |
| P07-04 | Isolation/creepage where applicable | Not applicable; no isolation barrier specified. | N/A |

# 8. Thermal & Reliability

| Ref | Review check | Comments / evidence | Disposition |
| --- | --- | --- | --- |
| P08-01 | Heat dissipation, thermal vias and copper | Regulator and LAN7800 copper areas reviewed; thermal test not recorded. | Closed |
| P08-02 | Mechanical robustness and strain relief | Through-hole connectors and mounting holes provide mechanical attachment. | Closed |
| P08-03 | High-current and high-voltage clearances | USB-powered low-voltage board; no high-voltage circuitry. | Closed |

# 9. Fabrication & Assembly

| Ref | Review check | Comments / evidence | Disposition |
| --- | --- | --- | --- |
| P09-01 | Solder mask, silkscreen and polarity marks | Silkscreen reference designators and assembly views prepared. | Closed |
| P09-02 | Assembly access, fiducials and test points | Assembly drawing covers top/bottom placement and notes. | Closed |
| P09-03 | BOM / CPL / footprint consistency | Manufacturing Rev05 includes BOM and placement records; release verification is separate. | Closed |
| P09-04 | Gerbers, drills and fabrication notes | Gerber/drill outputs and fabrication documents held in manufacturing pack. | Closed |
| P09-05 | DRC results and documented exceptions | Altium DRC report dated 17 Oct 2026: 0 warnings, 0 rule violations for enabled rules | Closed |

# Review summary

Items closed:  31 applicable checks (design owner confirmed resolved).

Items requiring action:  None recorded in this review.

Items not applicable:  4 checks, marked N/A in the tables.

Key decisions / changes:  Existing PCB layout and manufacturing documentation retained. No additional PCB changes directed by this review.

Evidence / files reviewed:  Rev A PCB layout in design review PDF; Altium DRC report (17 Oct 2026); assembly drawing; RevA manufacturing records; earlier review history.

Review conclusion:  PCB review complete. All applicable checks closed on design owner confirmation; no open review actions. Enabled Altium DRC rules reported 0 warnings and 0 violations; this is not manufacturing or test approval.

Reference documents: LAN7800 schematic sheets 01_USB_B_and_Power and 02_LAN7800_Controller; PCB design review PDF; Rev1.1/Rev1.3 review history; Altium DRC dated 17 October 2026; manufacturing RevA.