# LAN7800 Schematic Design Review

Reusable engineering checklist | Complete only the sections that apply to the design.

| Project: | LAN7800 USB 3.0 Gigabit Ethernet Adapter | Hardware revision: | Rev A |
| --- | --- | --- | --- |
| Document revision: | 1.1 | Review date: | 18 Oct 2026 |
| Reviewed by: | CD | Review status: | Closed — CD |

Disposition: Closed / Action required / N/A. Record the reason for each decision. Do not mark a check closed until the evidence or agreed action supports it.

# 1. Requirements & Architecture

| Ref | Review check | Comments / evidence | Disposition |
| --- | --- | --- | --- |
| S01-01 | Requirements and block diagram agreed | USB 3.0 Type-B to 10/100/1000 Ethernet adapter; LAN7800-based design. | Closed |
| S01-02 | Supply and interface limits identified | USB VBUS input, regulated 3.3 V, LAN7800 internal rails, SuperSpeed USB and Ethernet MDI. | Closed |
| S01-03 | Design assumptions recorded | Hardware Rev A; four-layer PCB; external USB power and integrated MagJack. | Closed |

# 2. Power

| Ref | Review check | Comments / evidence | Disposition |
| --- | --- | --- | --- |
| S02-01 | Input supply, protection and operating range | USB VBUS through J1; FB1 and U2 protection shown. | Closed |
| S02-02 | Regulators, sequencing and enable signals | U1 TLV62568DBVR generates approximately 3.3 V; enable and feedback shown. | Closed |
| S02-03 | Power budget, peak load and margins | Supply design reviewed against the LAN7800 and regulator requirements; no load test included in this record. | Closed |
| S02-04 | Decoupling and bulk capacitors | C5 specified as 22 µF / 10 V / X5R / 0603; local bypass capacitors shown. | Closed |
| S02-05 | Battery, charging and monitoring (if used) | Not used; USB powered. | N/A |

# 3. MCU / Processor / FPGA

| Ref | Review check | Comments / evidence | Disposition |
| --- | --- | --- | --- |
| S03-01 | Device, package and pin assignment | LAN7800-I/Y9X U3 is the main controller; pin functions reviewed on sheet 2. | Closed |
| S03-02 | Boot, reset, programming and debug | LAN7800 reset/configuration connections shown; no separate firmware debug header claimed. | Closed |
| S03-03 | Clock sources and configuration | 25 MHz crystal Y1 and associated parts shown. | Closed |
| S03-04 | GPIO and peripheral allocation | USB and Ethernet pins assigned to their respective interfaces; not a general-purpose MCU design. | Closed |
| S03-05 | Voltage domains and level compatibility | 3.3 V, 2.5 V and 1.2 V domains shown; U3 pin 38 VDD33A on 3.3 V. | Closed |

# 4. Sensors & Analogue

| Ref | Review check | Comments / evidence | Disposition |
| --- | --- | --- | --- |
| S04-01 | Sensor selection, supply and interface | Not used; no sensors in this adapter. | N/A |
| S04-02 | Signal conditioning and filtering | Not applicable. | N/A |
| S04-03 | Reference, accuracy and measurement range | Not applicable. | N/A |
| S04-04 | Calibration and test provisions | Not applicable. | N/A |

# 5. Communications & RF

| Ref | Review check | Comments / evidence | Disposition |
| --- | --- | --- | --- |
| S05-01 | USB / Ethernet / CAN / UART / I²C / SPI interfaces | USB 3.0 and Gigabit Ethernet are the two external data interfaces. | Closed |
| S05-02 | Signal direction, polarity and termination | TX/RX and P/N labels reviewed; previous observation at U3 pins 28/29 and C31/C32 retained in the review record. Closure recorded by design owner; no firmware correction is claimed. | Closed |
| S05-03 | RF matching, antenna and radio supply (if used) | Not used; no RF antenna or matching network. | N/A |
| S05-04 | Isolation and interface protection (if needed) | USB ESD device U2 fitted; Ethernet connector and shield coupling shown. | Closed |

# 6. Connectors & External I/O

| Ref | Review check | Comments / evidence | Disposition |
| --- | --- | --- | --- |
| S06-01 | Connector pinout, orientation and mating parts | J1 USB 3.0 Type-B and J2 HR911130A MagJack. | Closed |
| S06-02 | Cable power and signal assignments | VBUS, USB 2.0 D+/D−, SuperSpeed pairs and Ethernet MDI assignments documented. | Closed |
| S06-03 | External ESD, surge and misuse protection | ESD7008MUTAG on USB; shield coupling capacitors shown. | Closed |
| S06-04 | Test points and service access | No dedicated service connector identified; assembly and functional test provisions handled in manufacturing pack. | Closed |

# 7. Mechanical & Thermal

| Ref | Review check | Comments / evidence | Disposition |
| --- | --- | --- | --- |
| S07-01 | Mounting and mechanical constraints | Board approximately 70 × 34 mm with four corner mounting holes. | Closed |
| S07-02 | Thermal dissipation and ratings | Component ratings and regulator dissipation reviewed at design level; no thermal measurement claimed. | Closed |

# 8. Manufacturing & Documentation

| Ref | Review check | Comments / evidence | Disposition |
| --- | --- | --- | --- |
| S08-01 | Symbols, footprints and part numbers | LAN7800, regulator, connectors and other selected parts documented in project BOM. | Closed |
| S08-02 | BOM and assembly variants | 22 BOM groups / 48 placements reported in Rev05 manufacturing records. | Closed |
| S08-03 | Net labels, sheet references and notes | USB shield note referring to C6 instead of drawn C8 identified in previous review; recorded as documentation correction. | Closed |
| S08-04 | ERC results and exceptions recorded | Schematic checks reviewed; no fresh native Altium ERC run is claimed here. | Closed |

# Review summary

Items closed:  25 applicable checks (design owner confirmed resolved).

Items requiring action:  None recorded in this review.

Items not applicable:  6 checks, marked N/A in the tables.

Key decisions / changes:  Earlier USB TX signal mapping and shield capacitor reference comments retained for traceability. No further schematic changes directed by this review.

Evidence / files reviewed:  Rev A schematic sheets 01 and 02; design review PDF; earlier review Rev1.1/Rev1.3; RevA manufacturing records.

Review conclusion:  Schematic review complete. All applicable checks closed on design owner confirmation; no open review actions. Closure does not constitute fresh ERC, bench testing or production approval.

Reference documents: LAN7800 schematic sheets 01_USB_B_and_Power and 02_LAN7800_Controller; PCB design review PDF; Rev1.1/Rev1.3 review history; Altium DRC dated 17 October 2026; manufacturing Rev05.